GestureDeck: Real-Time Hand Gesture Recognition for Touchless Slide Navigation

Week 11 - Project Proposal and Initial Planning

1. Project Title and Problem Statement

Presentations typically rely on a keyboard, mouse, or handheld remote for slide navigation. These controls may interrupt the flow of discussion when a presenter moves away from the computer or needs both hands for a demonstration. A webcam-based gesture interface provides a possible alternative because it can translate deliberate hand poses into presentation commands without additional physical controllers.

This project proposes GestureDeck, a desktop application that will recognize selected hand gestures and use them to control a locally displayed slide presentation. The central question is whether a pretrained hand-gesture recognition model, combined with simple decision rules, can provide sufficiently accurate and responsive slide navigation in an ordinary indoor environment. The project will be limited to one user, one visible hand, and image-based slides to keep development practical for an individual student.

2. Objectives and Expected Outcomes

The general objective is to develop and evaluate a functional touchless slide navigation system using real-time computer vision. Specifically, the project aims to capture webcam frames and identify supported hand gestures; associate recognized gestures with slide navigation and a control-lock function; minimize unintended commands by checking confidence and gesture stability; and assess the application using recorded indoor trials and runtime measurements.

The expected output is an operational Python application that displays a local slideshow, responds to intentional hand gestures, and shows the current command or lock state. Evaluation records will document successful commands, recognition errors, unintended actions, and processing speed. These are intended project outcomes rather than results already obtained.

3. Proposed Computer Vision Approach and Algorithms

The study will adopt a development-and-testing approach using Python, OpenCV, and the pretrained MediaPipe Gesture Recognizer. OpenCV will acquire video frames from a webcam, after which the frames will be converted from BGR to RGB format for inference. MediaPipe provides hand landmark estimates and gesture classifications, allowing the application to use established visual features without training a large neural network from scratch. The initial configuration will process one hand at approximately 640 x 480 resolution, with performance adjusted after testing on the available Ryzen 5 3400G computer. GPU acceleration from the RX 580 is not required for the proposed baseline.

The initial command mapping will use Thumb_Up for the next slide, Thumb_Down for the previous slide, and Open_Palm to enable or disable gesture control. All three are included among MediaPipe's supported pretrained categories. The application will accept a command only when its prediction score exceeds an initial threshold, the gesture remains stable across successive frames, and a short cooldown has elapsed since the last action. A release or change of hand pose will be required before repeating a command. Thresholds and waiting intervals will be determined through pilot tests, rather than treated as proven settings.

The application will display slide images from a local folder and record recognized gestures and executed commands. Performance assessment will compare intended gestures with observed predictions and slide actions. Accuracy and a confusion matrix will describe classification behavior, while command success rate, unintended command count, response delay, and average frames per second will describe practical usability.

4. Initial Dataset or Image/Video Sources

The recognition component will use Google's publicly available MediaPipe Gesture Recognizer model bundle, which includes a hand landmark model and a gesture classifier. The project therefore does not require the collection of a large training dataset. Python implementation details and model-loading procedures will be taken from the official guide. The presentation content will consist of five to eight sample slides prepared by the student and exported as PNG or JPEG images.

A small evaluation dataset will be recorded with a webcam in the student's room. The proposed collection consists of 120 labeled trials: 30 Thumb_Up trials, 30 Thumb_Down trials, 30 Open_Palm trials, and 30 no-command trials involving neutral hand movement or an empty camera view. Recordings will include reasonable variations in lighting, background, and hand position. Ten trials from each category will be reserved for preliminary threshold adjustment, while the remaining twenty per category will be held for final testing. Recordings will be organized by session to reduce repeated-frame bias, and all files will be stored locally.

5. System Architecture or Workflow Diagram

Figure 1 presents the proposed workflow. A live camera frame will first pass through OpenCV preprocessing, followed by landmark estimation and gesture classification in MediaPipe. The command-validation stage will reject uncertain or brief predictions before a permitted action reaches the slide viewer. This separation distinguishes the pretrained recognition model from the application logic developed for GestureDeck.

Figure 1. Proposed workflow of the GestureDeck system.

6. Task Assignments for Group Members, if Applicable

This is an individual project, and all activities will be undertaken by one student. Responsibilities include literature review and planning, installation of the software environment, integration of webcam capture and the pretrained model, development of the slide viewer and command-validation logic, collection and labeling of test recordings, evaluation of system performance, and preparation of the final report and demonstration.

7. Initial Project Timeline

The work is planned across Weeks 11 to 16, with the recognition prototype completed before the presentation interface and evaluation procedures. This sequence prioritizes early confirmation that the model and webcam operate on the available computer.

| Schedule | Planned Activities |
| --- | --- |
| Week 11 | Finalize the problem, scope, proposal, selected gestures, and initial data collection procedure. |
| Week 12 | Configure Python and webcam input; download the pretrained model and verify live gesture recognition. |
| Week 13 | Develop the local image-based slideshow and connect recognized gestures to navigation actions. |
| Week 14 | Implement gesture stability checks, confidence filtering, lock state, and command cooldown; conduct pilot trials. |
| Week 15 | Collect final test trials and analyze accuracy, false commands, response delay, and frame rate. |
| Week 16 | Refine the application, complete the written documentation, and prepare the live demonstration. |

References

Google AI Edge. (n.d.). Gesture recognition task guide. https://developers.google.com/edge/mediapipe/solutions/vision/gesture_recognizer

Google AI Edge. (n.d.). Gesture recognition guide for Python. https://developers.google.com/edge/mediapipe/solutions/vision/gesture_recognizer/python

scikit-learn developers. (n.d.). Classification report (precision, recall, F1 score). https://scikit-learn.org/stable/modules/generated/sklearn.metrics.classification_report.html